| A - Plug |  |  |  |  | B - Plug |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Name | Function | Color |  |  | Name | Function | Color |
| 1 | DPO 2 | Fan relay control | grn |  | 1 | REF | CAS 1 | Wht/blu |
| 2 | AVI 4 | PPS 1 | Grn/wht |  | 2 | SYNC | CAS 2 | wht |
| 3 | IGN 1 | 1 Leading | Brwn |  | 3 | AVI 7 | Air Temp | Red/yel |
| 4 | IGN 2 | 2 Leading | Blu |  | 4 | AVI 8 | Coolant Temp | Red/wht |
| 5 | IGN 3 | 3 Leading | grn |  | 5 | REF- | CAS 1 | Wht/orng |
| 6 | IGN 4 | 1 Trailing | red |  | 6 | SYNC- | CAS 2 | Wht/grn |
| 7 | IGN 5 | 2 Trailing | yel |  | 7 | SPI 4 | Clutch SW | Blu/yel |
| 8 | IGN 6 | 3 Trailing | orng |  | 8 | SPI 1 | Flex | Brn/wht |
| 9 | 5v+ | Aux sen 5v | Yel/blu |  | 9 | SPI 2 |  |  |
| 10 | GND | BAT/CHASSIS | Blk |  | 10 | SPI 3 | VSS | Blu/wht |
| 11 | GND | BAT/CHASSIS | blk |  | 11 | 12v | Ecu/Inj Power | Yel/blk |
| 12 | 8v |  |  |  | 12 | AVI 6 | Oil Pressure | Yel/red |
| 13 | 12v ign | Not used |  |  | 13 | AVI 1 | Oil Temp | Yel/Red |
| 14 | AVI 10 | OMP Position | Grn/Yel |  | 14 | SG | Sensor Ground | Grn/yel |
| 15 | AVI 9 | Fuel Pressure | Grn/Blu |  | 15 | SG | TB Ground | Big Blu |
| 16 | AVI 2 | TPS 1 | Tan/wht |  | 16 | SG | Pedal Ground | Brn/wht |
| 17 | AVI 3 | TPS 2 | Blu/wht |  | 17 | IGN 7 | IAC | Blk/red |
| 18 | DPO1 | Tacho | Blu |  | 18 | IGN 8 |  |  |
| 19 | INJ 1 | INJ 1 (pri F) | Brwn |  | 19 | DPO 4 |  |  |
| 20 | INJ 2 | INJ 2 (pri R) | Red |  | 20 | AVI 5 | PPS 2 | Orng/wht |
| 21 | INJ 3 | INJ 3 (sec F) | Blu |  | 21 | KNOCK 1 | KNOCK | wht |
| 22 | INJ 4 | INJ 4 (sec R) | Yel |  | 22 | KNOCK 2 |  |  |
| 23 | DPO 3 | Boost Solenoid | Orng/blk |  | 23 | CAN HI | CAN to aim | Wht/blu |
| 24 | DPO 5 | Fuel Pump Cont. | Grn/blu |  | 24 | CAN LO | CAN to aim | Wht |
| 25 | DPO 6 | Not used |  |  | 25 | DBW 1 | Motor 1 | Purple |
| 26 | 12v | Ecu/Inj Power | Yel/blk |  | 26 | DBW 2 | Motor 2 | Org/Blk |
| 27 | INJ 5 | INJ 5 (3 pri) | Grn |  |  |  |  |  |
| 28 | INJ 6 | INJ 6 (3 sec) | Orng |  |  |  |  |  |
| 29 | INJ 7 | FP PWM | purple |  |  |  |  |  |
| 30 | INJ 8 |  |  |  |  |  |  |  |
| 31 | Stepper 1 | OMP B1 | Grn/blk |  |  |  |  |  |
| 32 | Stepper 2 | OMP B2 | Blk/red |  |  |  |  |  |
| 33 | Stepper 3 | OMP A1 | Blk/yel |  |  |  |  |  |
| 34 | Stepper 4 | OMP A2 | Blu/blk |  |  |  |  |  |

Haltec h 2500 Elite 20b

| Throttle Body |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pin | Function | Haltech Pin | Name | Wire |
| A | Motor 2 | B26 | DBW 2 | Orng/Blk |
| B | Motor 1 | B25 | DBW 1 | Purple |
| C | TB Grnd | B15 | Sig Grnd | Big Blu |
| D | TPS 1 | A16 | AVI 2 | Tan/Wht |
| E | +5v | A9 | 5v | Yel/blu¦Pink |
| F | TPS 2 | A17 | AVI 3 | Blu/wht |

| Accelerator Pedal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pin | Function | Haltech Pin | Name | Wire |
| A | Ground | B16 | Sig Grnd | Brn/Wht |
| B | PPS 2 | B20 | AVI 5 | Orng/Wht |
| C | +5v | A9 | +5v | Yel/blu¦Pink |
| D | Ground | B16 | Sig Grnd | Brn/Wht |
| E | PPS1 | A2 | AVI 4 | Grn/wht |
| F | +5v | A9 | +5v | Yel/blu¦Pink |